| Dept/Location: M5 Prøvestenen | Assessment date: 21.02.2024 | Due for review: 21.02.2025 |
| --- | --- | --- |
| Assessment prepared by Andreas Kallmeyer Bloch | Approved by: | Reviewed by: |

| HAZARDS Risks detected | CAUSE Description of the activity assessed | Effect if the risks result in an accident | EXISTING CONTROL MEASURES Activities that reduces or eliminate the risk | FURTHER ACTION TO CONTROL RISK Activity that reduces or eliminate the risk | Residual Risk - after introduction of the risk reducing activity | Person responsible - for introducing risk reducing activity | FREQUENCY Likelihood of the hazard (H,M,L) | SEVERITY Severity of the hazard (H,M,L) | RISK EVALUATION (C,SC,U) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diving from vessel |  |  |  |  |  |  |  |  |  |
| Man over board in case of high waves or strong current, it can be difficult to find the person that has fallen over board | Poor weather. Use of work boats during dark hours | Drowning Dehydration Hypothermia | Use of correct PPE and high visibility cloth Flotation suit or PFD VHF Radio communication | Emergency procedure 2 person operation at night or adverse weather Strobe light Toolbox talk | Weather depend Human error | Master | L | L | C |
| Contact with moving parts | Use of crane or use of anchor winch | Soft body parts injured Broken body parts | Monitoring of the working area by operator during use of crane or anchor winch | Instruction of personnel. Toolbox talk | Human error Malfunction equipment | Master | M | L | SC |
| Hypothermia | Temperature extremes Falling overboard | Shivering and mental confusion Clinical death Brain death | MOB always prepared and tested at least once a week. Regular MOB drills Survival packs on board the vessel including sleeping bags, additional survival suits | Monitor transfer of personnel. Audit survival kits regularly according to vessels ISM system. Regular drills according to ships rolls | Human Error | Master | M | M | SC |
| Fire on board | Electrical failure, equipment failure, flammable liquids | Burns, death, vessel loss | Fire fighting equipment. Trained crew, life raft | MOB drills, inspection of equipment Daily checks No smoking near flammable liquids | Human Error Malfunction of equipment | Master | L | M | SC |
| Noise | Running of engines, generators and equipment | Hearing damage | Wearing of ear muff while equipment running |  | Human Error | Master | M | L | SC |
| Fumes/ dangerous gases | Exhaust fumes from generators and engines | Poisoning | All exhausts to atmosphere | Monitor enclosed spaces, ventilate Toolbox talk | Human Error Malfunction of equipment | Master | L | M | SC |
| Collision with other vessel | Location in channel, poor visibility | Injury Death Drowning | Competent Master and crew. Vessel Lights. Navigation equipment Plans and procedures | Monitor AIS. Lookout. Monitor ch16 | Human Error Malfunction of equipment | Master | L | M | SC |
| Electric shock / burns | Use of 240v electrical equipment interface with utilities | Internal / external burns, death, heart attack | Permit to Work System Regular testing of portable electrical equipment. | Maintenance of equipment | Human error Malfunction equipment | Master | L | H | SC |
| Contact with hazardous substances | Contact with oils, diesel and greases. Fuel storage Batteries | Skin irritation | Minimization on contact Use of personal protective equipment | Instruction of personnel. Toolbox talk | Human error | Master | M | L | SC |
| Slips, trips and falls | Fall on deck | Broken body parts, soft part injury, death | Keep the deck clean and tidy – secure a clean deck | Instruction of personnel. Toolbox talk | Human error | Master | M | M | SC |
| Lacerations / other first aid injuries and illness | Use of hand tools Rough edges Illness Slips / trips | Body injuries, pass out | First aid equipment on the vessel PPE | Instruction of personnel. Toolbox talk | Human error | Master | L | L | C |
| Failure of Personal Protective Equipment | Not regularly inspecting PPE Weather damage Wear and Tear | Body injuries, drowning | Planned maintenance | Regularly inspection on PPE | Human error | Master | L | L | C |
| Failure of Fire Extinguishers | Not regularly inspecting equipment | Burns, death, vessel loss | Programme for inspection and replacement of extinguishers | Trained crew, life raft | MOB drills, inspection of equipment Daily checks No smoking near flammable liquids | Master | L | L | C |
| Basis for safety decision is wrong | Lack of training and information Poor handover at crew change | No basic knowledge | Ensure familiarization with handover procedure | Verification for complete handover | Human error | Master | L | L | C |
| Squeeze of fingers / arms Injury to body | Launch of workboats/ribs | Soft parts injured | Use of correct PPE Competent deckhands | Toolbox talk | Weather depend | Master | L | L | C |
| Fall down | Lose one’s balance Careless | Body injuries | Use lifejacket, lifeline At winter use Survival suit | Trained crew Toolbox talk | Human error Weather depend | Master | L | M | SC |

RISK EVALUATION: C (Controlled Risk); SC (Risk Semi-controlled); U (Risk uncontrolled)

## RISK EVALUATION CRITERIA

|  |  | PROBABILITY OF THE HAZARD |  |  |
| --- | --- | --- | --- | --- |
|  |  | HIGH | MEDIUM | LOW |
|  | HIGH |  |  |  |
| # SEVERITY OF THE HAZARD | MEDIUM |  |  |  |
|  | LOW |  |  |  |

PROBABILITY RATINGS:

HIGH = likely the hazard will occur

MEDIUM = possible the hazard will occur

LOW = unlikely the hazard will occur

SEVERITY RATINGS:

HIGH = fatality/loss of limbs/serious illness permanent

MEDIUM = broken bones/minor illness temporary

LOW = scratch/bruise/mild health effect

RISK EVALUATION RESULTS:

UNCONTROLLED:	RED (control measures required)	If printing black & white, Darkest Grey

SEMICONTROLLED: YELLOW (control measures required)	If printing black & white, Lightest Grey

CONTROLLED:  GREEN (no further control measures required) If printing black & white, Medium Grey